Porous Pavement Example

For this example, we will start with the model file we created for No Controls and add Porous Pavement.  Only the parking stalls in the Parking Lot Source Area will be treated by the Porous Pavement.

Open the No Controls model file and Save the File with a new name.

Change the Site Description in the Current File Data to reflect the Porous Pavement.

Porous Pavement is only being applied to 1.46 acres of the parking lot in paved parking 1 source area (having a total of 2.85 acres). Select the PP (porous pavement) control as the first control practice for the paved parking 1 area. Porous pavement is only available as the first control practice in source areas, while other controls can be located as the second practice for any overflow.

Use the Pull Down menu next to the Parking Lot Source Area to select the Porous Pavement control measure.

Enter the data shown below.  Only the parking stalls will have Porous Pavement (1.46 acres), and this setup assumes that runoff from the remaining parking lot area drains towards this porous pavement area.

Note: when moving through the Porous Pavement form, press the “Enter” key to move to the next cell, not the “Tab” key.

Run the model.

Results

Runoff Volume without controls:			173,467 cu ft

Outfall Total with controls:			68,036 cu ft

Runoff Volume Percent Reduction:		61%

Particulate Solids Concentration (with controls):	38 mg/L

Particulate Solids Yield (with controls):		177 lbs

Particulate Solids Percent Reduction:		77%

Rv (with controls):				0.17

Approx. Urban Stream Classification:		Fair

Total Phosphorus Concentration (with controls):	1.3

Total Phosphorus Yield (with controls):		5.4 lbs

Total Phosphorus Yield Percent Reduction:	18 %

Annualized Value of all costs:			$11,949

The pollution reduction reported at the outfall is the overall pollution reduction for the entire site.

To see the pollution reduction from just the Porous Pavement, select the “Control Practices” tab.

The Porous Pavement is reducing the runoff volume and the TSS load by 100%for the complete Parking Lot, even with the runon from the area with conventional pavement.