Catcbasin Example

For this example, we will start with the model file we created for No Controls and add Catchbasins to treat runoff from the parking lot.

Open the No Controls model file and Save the File with a new name.

Change the Site Description in the Current File Data to reflect the Catchbasins use.

The Catchbasins will be modeled as Source Area Controls located in the paved parking 1 source area (2.85 acres). Select the catchbasins in the “first control practice” column:

Select the Catch Basin control measure from the Source Area Control Measure pull-down menu next to the appropriate Parking Lot source area. This will open the Catch Basin form.

Enter the data shown below.

There are three 2’ x 3’ catchbasins with a 2 foot sump below the invert of the outlet storm sewer.  The catchbasin sumps are in the inlets to the storm sewer.  We are assuming each catchbasin treats its own area – therefore the catchbasins are in “parallel”, not in series.  If the catchbasins were in series, the map would need to be set up differently.

Note: when moving through the Catchbasin form, press the “Enter” key to move to the next cell, not the “Tab” key.

Run the model.

Results

Runoff Volume without controls:			173,467 cu ft

Outfall Total with controls:			173,467 cu ft

Runoff Volume Percent Reduction:		0%

Particulate Solids Concentration (with controls):	52 mg/L

Particulate Solids Yield (with controls):		620 lbs

Particulate Solids Percent Reduction:		19%

Rv (with controls):				0.43

Approx. Urban Stream Classification:		Poor

Total Phosphorus Concentration (with controls):	0.59

Total Phosphorus Yield (with controls):		6.4 lbs

Total Phosphorus Yield Percent Reduction:	1.6 %

Annualized Value of all costs:			$1,178

The pollution reduction reported at the outfall (on the Output Summary Tab) is the overall pollution reduction for the entire site. Review the Control Practices tabs to see the pollution reduction for the catchbasin controls at the paved parking area:

The catchbasins are reducing the pollution load from their Parking Lot sources by 25.4%, which was reduced to about 19% for the total area load (most of the particulate solids were originating from the parking lot, so these are relatively close). As noted here, the catchbasin sumps were only about 8% full after one year so more frequent cleaning would not benefit the effluent water quality. In addition, catchbasins do not have any water volume reductions associated with their use (in some areas, catchbasins are constructed to infiltrate through their bottom and would then act as dry wells with volume reductions; there is an option on the catchbasin form to describe the “leakage” rate through the bottom of the catchbasin, but this should only be used after groundwater contamination potential evaluations consider this to be a safe practice).