Biofilter Example

For this example, we will start with the model file we created for No Controls and add Biofilters.  We will assume all of the source areas in the Commercial 1 land use will be routed to the Biofilters.

Open the No Controls model file and Save the File with a new name.

Change the Site Description in the Current File Data to reflect the Biofilters.

Disconnect the map and add the Biofilter control measure.  Reconnect the map to make a contiguous network to the Outfall.

Double click on the Biofilter label to access the Biofilter form.  All the “Small Landscaped 2” Source Areas will be Biofilters.  We will enter the data for a “typical” Biofilter and then enter the number of Biofilters.

Note: when moving through the Biofilter form, press the “Enter” key to move to the next cell, not the “Tab” key.

The information the model needs to evaluate the Engineered Soil, or Media, in the Biofilter can be entered using default values or, as User Defined values.

Enter the fraction for each soil type.  Then select “Apply All Values”.  Select “Continue”.

When the boxes next to “Apply” are checked, the program will use the calculated values in the Biofilter form.  The values will not appear until an Engineered Soil Depth is entered.

Run the model.

Results

Runoff Volume without controls:			173,467 cu ft

Outfall Total with controls:			63,115

Runoff Volume Percent Reduction:		63%

Particulate Solids Concentration (with controls):	37 mg/L

Particulate Solids Yield (with controls):		145 lbs

Particulate Solids Percent Reduction:		79%

Rv (with controls):				0.16

Approx. Urban Stream Classification:		Fair

Total Phosphorus Concentration (with controls):	0.55

Total Phosphorus Yield (with controls):		2.19 lbs

Total Phosphorus Yield Percent Reduction:	66.4 %

Annualized Value of all costs:			$15,906

The pollution reduction reported at the outfall is the overall pollution reduction for the entire site.

To see the pollution reduction from just the Biofilters, select the “Control Practices” tab.

The Biofilters are reducing the runoff volume by 63.6% and the TSS load by 79.3% from the source areas in the Commercial 1 land use.

ET monitoring stations example (from Pitt, et al. 2009).

(use Oakland foothills CIMIS station 244)

ETo California Irrigation Management Information System (CIMIS)

| Station Map ID | State | Lat | Long | Elev | Station Name | CIMIS Average Monthly Rates (ETo) (inday) |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Years of Data | Jan | Feb | Mar | Apr | May | Jun | Jul | Aug | Sep | Oct | Nov | Dec |
| 233 | CA | 35.603111 | -119.212586 | 415 | Famoso | N/A | 0.05 | 0.08 | 0.12 | 0.17 | 0.22 | 0.26 | 0.28 | 0.25 | 0.19 | 0.13 | 0.07 | 0.05 |
| 234 | CA | 38.501258 | -121.978528 | 136 | Winters | N/A | 0.03 | 0.06 | 0.11 | 0.17 | 0.21 | 0.25 | 0.26 | 0.22 | 0.17 | 0.11 | 0.05 | 0.03 |
| 235 | CA | 38.116125 | -121.659214 | -1 | Twitchell Islan | N/A | 0.05 | 0.08 | 0.12 | 0.17 | 0.22 | 0.26 | 0.28 | 0.25 | 0.19 | 0.13 | 0.07 | 0.05 |
| 236 | CA | 33.536894 | -115.992803 | -180 | Mecca | N/A | 0.05 | 0.09 | 0.13 | 0.19 | 0.25 | 0.29 | 0.30 | 0.27 | 0.21 | 0.14 | 0.08 | 0.05 |
| 237 | CA | 36.721083 | -119.389028 | 450 | Orange Cove | N/A | 0.03 | 0.06 | 0.11 | 0.16 | 0.22 | 0.26 | 0.28 | 0.25 | 0.19 | 0.11 | 0.06 | 0.02 |
| 238 | CA | 36.822861 | -121.467869 | 245 | San Juan Valley | N/A | 0.06 | 0.08 | 0.12 | 0.16 | 0.17 | 0.19 | 0.18 | 0.17 | 0.14 | 0.11 | 0.08 | 0.06 |
| 239 | CA | 38.266428 | -122.616464 | 97 | Petaluma | N/A | 0.03 | 0.06 | 0.09 | 0.14 | 0.18 | 0.21 | 0.21 | 0.19 | 0.15 | 0.10 | 0.05 | 0.03 |
| 240 | CA | 37.016528 | -120.186394 | 230 | Madera | N/A | 0.05 | 0.09 | 0.13 | 0.19 | 0.25 | 0.29 | 0.31 | 0.28 | 0.21 | 0.13 | 0.07 | 0.04 |
| 241 | CA | 35.505833 | -119.691144 | 410 | Belridge | N/A | 0.05 | 0.08 | 0.12 | 0.17 | 0.22 | 0.26 | 0.28 | 0.25 | 0.19 | 0.13 | 0.07 | 0.05 |
| 242 | CA | 32.628208 | -116.939281 | 580 | Otay Lake | N/A | 0.04 | 0.07 | 0.11 | 0.16 | 0.19 | 0.23 | 0.25 | 0.22 | 0.17 | 0.11 | 0.07 | 0.04 |
| 243 | CA | 37.314139 | -120.386700 | 200 | Merced | N/A | 0.04 | 0.07 | 0.11 | 0.17 | 0.22 | 0.26 | 0.26 | 0.23 | 0.18 | 0.12 | 0.06 | 0.03 |
| 244 | CA | 37.780653 | -122.180150 | 145 | Oakland Foothill | N/A | 0.04 | 0.06 | 0.10 | 0.15 | 0.18 | 0.21 | 0.22 | 0.20 | 0.15 | 0.11 | 0.05 | 0.04 |
| 245 | CA | 32.885847 | -117.143142 | 445 | Miramar | N/A | 0.06 | 0.08 | 0.11 | 0.15 | 0.17 | 0.19 | 0.19 | 0.18 | 0.15 | 0.11 | 0.08 | 0.06 |
| 246 | CA | 33.532222 | -114.633889 | 251 | Ripley | N/A | 0.08 | 0.12 | 0.17 | 0.23 | 0.28 | 0.32 | 0.31 | 0.28 | 0.23 | 0.16 | 0.10 | 0.07 |
| 247 | CA | 34.219386 | -118.992439 | 130 | Camarillo | N/A | 0.06 | 0.08 | 0.11 | 0.15 | 0.17 | 0.19 | 0.19 | 0.18 | 0.15 | 0.11 | 0.08 | 0.06 |
| 248 | CA | 33.081050 | -116.975697 | 390 | Escondido SPV | N/A | 0.09 | 0.10 | 0.12 | 0.18 | 0.20 | 0.23 | 0.23 | 0.22 | 0.19 | 0.13 | 0.11 | 0.08 |
| 249 | CA | 38.599158 | -121.540406 | 40 | Bryte | N/A | 0.03 | 0.06 | 0.11 | 0.17 | 0.21 | 0.25 | 0.26 | 0.22 | 0.17 | 0.11 | 0.05 | 0.03 |
| 250 | CA | 34.233639 | -119.196922 | 48 | Oxnard | N/A | 0.06 | 0.08 | 0.11 | 0.15 | 0.17 | 0.19 | 0.19 | 0.18 | 0.15 | 0.11 | 0.08 | 0.06 |
| 251 | CA | 37.995947 | -122.466308 | 5 | Point San Pedro | N/A | 0.04 | 0.06 | 0.10 | 0.14 | 0.17 | 0.21 | 0.21 | 0.19 | 0.14 | 0.09 | 0.04 | 0.03 |
| 252 | CA | 38.419439 | -122.658719 | 270 | Bennett Valley | N/A | 0.03 | 0.05 | 0.09 | 0.14 | 0.17 | 0.20 | 0.20 | 0.19 | 0.14 | 0.10 | 0.05 | 0.03 |
| 253 | CA | 34.146372 | -117.985797 | 595 | Monrovia | N/A | 0.05 | 0.08 | 0.12 | 0.17 | 0.22 | 0.26 | 0.28 | 0.25 | 0.19 | 0.13 | 0.07 | 0.05 |
| 254 | CA | 35.335261 | -120.735881 | 285 | San Luis Obispo West | N/A | 0.07 | 0.09 | 0.12 | 0.17 | 0.18 | 0.21 | 0.21 | 0.20 | 0.16 | 0.13 | 0.10 | 0.07 |
| 255 | CA | 37.438944 | -121.138511 | 183 | Patterson | N/A | 0.05 | 0.08 | 0.12 | 0.17 | 0.22 | 0.26 | 0.28 | 0.25 | 0.19 | 0.13 | 0.07 | 0.05 |
| 256 | CA | 35.472556 | -120.648142 | 885 | Atascadero | N/A | 0.07 | 0.09 | 0.12 | 0.17 | 0.18 | 0.21 | 0.21 | 0.20 | 0.16 | 0.13 | 0.10 | 0.07 |
| 257 | CA | 34.841878 | -120.212736 | 536 | Sisquoc | N/A | 0.06 | 0.09 | 0.12 | 0.17 | 0.19 | 0.20 | 0.19 | 0.18 | 0.15 | 0.11 | 0.08 | 0.06 |
| 258 | CA | 38.129933 | -121.386594 | 25 | Lodi West | N/A | 0.03 | 0.06 | 0.11 | 0.17 | 0.21 | 0.25 | 0.25 | 0.22 | 0.17 | 0.11 | 0.05 | 0.02 |
| 259 | CA | 37.725881 | -121.475517 | 82 | Tracy | N/A | 0.03 | 0.06 | 0.11 | 0.18 | 0.22 | 0.26 | 0.27 | 0.23 | 0.18 | 0.12 | 0.06 | 0.03 |
| 260 | CA | 36.082056 | -119.093422 | 400 | Porterville | N/A | 0.03 | 0.07 | 0.12 | 0.18 | 0.23 | 0.27 | 0.27 | 0.24 | 0.19 | 0.13 | 0.06 | 0.03 |
| 261 | CA | 38.015372 | -122.020278 | 35 | Concord | N/A | 0.03 | 0.06 | 0.11 | 0.18 | 0.22 | 0.26 | 0.27 | 0.23 | 0.18 | 0.12 | 0.06 | 0.03 |
| 262 | CA | 37.598758 | -122.053233 | 16 | Union City | N/A | 0.05 | 0.07 | 0.11 | 0.16 | 0.17 | 0.21 | 0.22 | 0.19 | 0.15 | 0.11 | 0.06 | 0.05 |
| 263 | CA | 32.901867 | -117.250458 | 335 | Torrey Pines | N/A | 0.06 | 0.08 | 0.11 | 0.15 | 0.17 | 0.19 | 0.19 | 0.18 | 0.15 | 0.11 | 0.08 | 0.06 |
| 264 | CA | 33.798697 | -118.094792 | 17 | Long Beach | N/A | 0.05 | 0.08 | 0.12 | 0.16 | 0.17 | 0.19 | 0.19 | 0.19 | 0.15 | 0.10 | 0.07 | 0.05 |
| 265 | CA | 33.383697 | -114.719211 | 230 | Palo Verde II | N/A | 0.08 | 0.12 | 0.18 | 0.24 | 0.28 | 0.31 | 0.31 | 0.28 | 0.22 | 0.15 | 0.10 | 0.07 |
| 266 | CA | 37.837614 | -122.140739 | 510 | Moraga | N/A | 0.03 | 0.05 | 0.09 | 0.15 | 0.18 | 0.22 | 0.24 | 0.20 | 0.16 | 0.11 | 0.05 | 0.03 |
| 267 | CA | 33.663325 | -117.093383 | 1626 | Winchester | N/A | 0.09 | 0.10 | 0.12 | 0.18 | 0.20 | 0.23 | 0.23 | 0.22 | 0.19 | 0.13 | 0.11 | 0.08 |