Cost-Effectiveness Evaluation Example

The purpose of this example is to show you how to use the cost analysis and batch processing options in WinSLAMM to evaluate the cost-effectiveness of stormwater control practices, including performance (effluent quality, percent reduction, life-cycle costs, including maintenance costs, etc.). For this example, we will use the previously created files for the individual controls, along with combinations of controls to identify the most cost efficient set of controls.

1. Select the Cost Analysis Template.

Go to the ‘Tools/Access Cost Data’ menu and press the ‘Select File’ button, and select the file:  c:\WinSLAMM Files\Birmingham Cost Data.csv. Below is a screen shot of this summary data form:

In this example, Birmingham was selected having a city engineering cost index of 6376.5, which 0.7 times the national average. The land costs by land use are shown on this table and can be easily changed, along with the interest rate and the project life. You will notice that there are tabs describing the unit costs for many of the types of stormwater controls being examined (some, such as porous pavement, have no land costs associated with them). Below is the example for biofiltration devices (which covers many types of infiltration controls). The pre-determined costs are equation coefficients from the referenced cost studies and reflect the range of costs associated with the length of the biofilter for different depths, along with the unit cost for the crushed stone. It is also possible to insert other local unit cost rates for different components of the biofilter, based on the volume of the excavation (yd3), the length (ft), surface are (yd2), or per unit. Other tabs have similar sets of information. The model does not include any costs for cisterns/water tanks yet, so those will have to be manually added if they are included in your evaluation.

2. Create the .mdb Files

The individual files for the commercial area were described in the earlier modules and included descriptions for different control practices: grass swales, water tanks, porous pavement, and biofilters. It is easy to create additional files having combinations of these controls, as illustrated in the list below:

It is also possible to create files having different sizes or use of the controls using the simple multiplier options for each control, or by changing the cleaning frequency (for catchbasins and street cleaning).

3. Process the Set of Files

- Go to the “Run/Set of Project Files” menu item.

- Browse to the folder where the project files are located and press “OK”.

- The window that allows you to run or modify a set of .mdb files will appear. In the left hand box, double-click on the files you want in the analysis. You can move them all, if desired.

- Click on the base condition file (no management conditions.mdb).  Click on the arrow pointing up to move this base condition file to the top of the box in the Run List box. You can also re-arrange the order of any of the other files listed (as I did here to indicate location in the area). All of the files must include the same rain series and have the same areas if compared to a base file.

- Check the box with the label “First File is Base Condition”. The will allow the program to calculate the percent reductions of each of the files compared to the base condition file. It is also possible to sort the files by selecting a file and using the up or down arrow to move it (this helps sort the basic spreadsheet order, but they can also be sorted after the analyses in the spreadsheet)

- Check the box “Include Cost Estimates”.

- Press the button “Select Cost Data File” and browse to and select the appropriate cost file:  Birmingham Cost Data.csv.  The illustration below shows how the form should look after you have completed these actions.

Press the “Run Files in Run List” button.  The “view output file” button will display an output summary as shown below (partial data set shown here):

The program will also create two summary output *.csv spreadsheet files.  The summary output files are:

- MDB_SetOutput.csv – This file summarizes the output for each file in a one-line-per-event format.  The output is illustrated below for part of the spreadsheet file. The spreadsheet can be used to manipulate the information and create plots, as shown below:

The most cost-effective stormwater control programs for volume reduction, based on these analyses, would be (the tank costs are calculated to be zero in this example so those costs would need to be added):

- Porous pavement (half) with 25 water tanks, with a maximum reduction of 75% at a cost of $0.02/ft3 removed (not including cost savings associated with not needed alternative pavement)

- Biofilters with 5 tanks and half porous pavement, with a maximum reduction of 86% at a cost of $0.03/ft3 removed

- Biofilters with 5 tanks and porous pavement, with a maximum reduction of 91% at a cost of $0.04/ft3 removed

The most cost-effective stormwater control programs for particulate solids reductions would be (the tank costs are calculated to be zero in this example so those costs would need to be added):

- Grass swales, with a maximum reduction of 53% at a cost of $3.20/lb removed (not including the cost savings associated with no needed other drainage components)

- Biofilters, with a maximum reduction of 81% at a cost of $4.55/lb removed

- Biofilters with 5 tanks and half porous pavement, with a maximum reduction of 95% at a cost of $6.69/lb removed

The most cost-effective stormwater control programs for total phosphorus reductions would be (the tank costs are calculated to be zero in this example so those costs would need to be added):

Grass swales, with a maximum reduction of 45% at a cost of $199/lb removed

Biofilters, with a maximum reduction of 69% at a cost of $282/lb removed

Biofilters with 5 water tanks and half porous pavement, with a maximum reduction of 90% at a cost of $369/lb removed

Partial Data Included in MDB_SetOutput.csv File:

|  | Runoff Volume (cf) | Model Run Percent Runoff Volume Reduction | Rv | Biological Condition | Particulate Solids Yield (lbs) | Model Run Particulate Solids Yield Percent Reduction | Particulate Solids Concentration (mg/L) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| No Management Conditions | 3,181,309 | n/a | 0.43 | Poor | 13,439 | n/a | 68 |
| Water tanks for roof runoff | 3,047,417 | 4.2 | 0.41 | Poor | 13,389 | 0.4 | 70 |
| Water tanks for roof runoff 5 tanks | 2,949,608 | 7.3 | 0.40 | Poor | 13,352 | 0.7 | 73 |
| Water tanks for roof runoff 25 tanks | 2,836,237 | 10.9 | 0.39 | Poor | 13,310 | 1.0 | 75 |
| Water tanks for roof runoff 75 tanks | 2,685,061 | 15.6 | 0.37 | Poor | 13,253 | 1.4 | 79 |
| Porous pavement | 999,922 | 68.6 | 0.14 | Fair | 5,656 | 57.9 | 91 |
| Porous pavement and 5 tanks | 766,367 | 75.9 | 0.10 | Good | 5,569 | 58.6 | 116 |
| Porous pavement half and 25 tanks | 809,604 | 74.6 | 0.11 | Good | 5,588 | 58.4 | 111 |
| Biofilters | 2,222,526 | 30.1 | 0.30 | Poor | 2,497 | 81.4 | 18 |
| Biofilters and 5 tanks | 2,019,277 | 36.5 | 0.27 | Poor | 2,374 | 82.3 | 19 |
| Biofilters 5 tanks and half porous pavement | 444,249 | 86.0 | 0.06 | Good | 626 | 95.3 | 23 |
| Biofilters 5 tanks and porous pavement | 297,065 | 90.7 | 0.04 | Good | 554 | 95.9 | 30 |
| Grass swales | 2,540,534 | 20.1 | 0.35 | Poor | 6,313 | 53.0 | 40 |

2. MDBSetOutputCostDetail.csv – This file provides cost detail information for each control practice in the set of files that were analyzed. An example is shown below (only for the data columns relevant to the stormwater controls evaluated in this example) (the tank costs are calculated to be zero in this example so those costs would need to be added):

| File Name | File Number | Control Practice Type | Control Practice Cost Variable | Control Practice Cost Variable Value | Capital Cost | Land Cost | Maintenance Cost | Present Value Multiplier | Present Value of All Costs | Annual Value Multiplier | Annual Value of All Costs |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Biofilters | 7 | BF | BF Length (ft) | 27.5 | 68,687 | 4,982 | 4,095 | 12.46 | 124,697 | 0.08 | 10,006 |
| Biofilters | 7 | BF | BF Length (ft) | 27.5 | 68,687 | 4,982 | 4,095 | 12.46 | 124,697 | 0.08 | 10,006 |
| Biofilters | 7 | BF | BF Length (ft) | 27.5 | 68,687 | 4,982 | 4,095 | 12.46 | 124,697 | 0.08 | 10,006 |
| Biofilters | 7 | BF | BF Length (ft) | 27.5 | 68,687 | 4,982 | 4,095 | 12.46 | 124,697 | 0.08 | 10,006 |
| Porous pavement | 5 | PP | Stone Depth (ft) | 1 | 172,867 | 0 | 579 | 12.46 | 180,077 | 0.08 | 14,450 |
| Porous pavement | 5 | PP | Stone Depth (ft) | 1 | 172,867 | 0 | 579 | 12.46 | 180,077 | 0.08 | 14,450 |
| Porous pavement and 5 tanks | 6 | PP | Stone Depth (ft) | 1 | 172,867 | 0 | 579 | 12.46 | 180,077 | 0.08 | 14,450 |
| Porous pavement | 5 | PP | Stone Depth (ft) | 1 | 172,867 | 0 | 579 | 12.46 | 180,077 | 0.08 | 14,450 |
| Porous pavement half and 25 tanks | 7 | PP | Stone Depth (ft) | 1 | 86,433 | 0 | 289 | 12.46 | 90,038 | 0.08 | 7,225 |
| Biofilters | 8 | BF | BF Length (ft) | 27.5 | 68,687 | 4,982 | 4,095 | 12.46 | 124,697 | 0.08 | 10,006 |
| Biofilters | 8 | BF | BF Length (ft) | 27.5 | 68,687 | 4,982 | 4,095 | 12.46 | 124,697 | 0.08 | 10,006 |
| Biofilters and 5 tanks | 9 | BF | BF Length (ft) | 27.5 | 68,687 | 4,982 | 4,095 | 12.46 | 124,697 | 0.08 | 10,006 |
| Biofilters | 8 | BF | BF Length (ft) | 27.5 | 68,687 | 4,982 | 4,095 | 12.46 | 124,697 | 0.08 | 10,006 |
| Biofilters 5 tanks and half porous pavement | 10 | PP | Stone Depth (ft) | 1 | 86,433 | 0 | 289 | 12.46 | 90,038 | 0.08 | 7,225 |
| Biofilters 5 tanks and half porous pavement | 10 | BF | BF Length (ft) | 27.5 | 68,687 | 4,982 | 4,095 | 12.46 | 124,697 | 0.08 | 10,006 |
| Biofilters | 8 | BF | BF Length (ft) | 27.5 | 68,687 | 4,982 | 4,095 | 12.46 | 124,697 | 0.08 | 10,006 |
| Biofilters 5 tanks and porous pavement | 11 | PP | Stone Depth (ft) | 1 | 172,867 | 0 | 579 | 12.46 | 180,077 | 0.08 | 14,450 |
| Biofilters 5 tanks and porous pavement | 11 | BF | BF Length (ft) | 27.5 | 68,687 | 4,982 | 4,095 | 12.46 | 124,697 | 0.08 | 10,006 |
| Grass swales | 12 | GS | Swale Depth (ft) | 2 | 25,234 | 18,352 | 1,086 | 12.46 | 57,115 | 0.08 | 4,583 |