Biofilter Example

For this example, we will start with the model file we created for No Controls and add Biofilters.  We will assume all of the source areas in the Commercial 1 land use will be routed to the Biofilters.

Open the No Controls model file and Save the File with a new name.

Change the Site Description in the Current File Data to reflect the Biofilters.

Disconnect the map and add the Biofilter control measure.  Reconnect the map to make a contiguous network to the Outfall.

Double click on the Biofilter label to access the Biofilter form.  All the “Small Landscaped 2” Source Areas will be Biofilters.  We will enter the data for a “typical” Biofilter and then enter the number of Biofilters.

Note: when moving through the Biofilter form, press the “Enter” key to move to the next cell, not the “Tab” key.

The information the model needs to evaluate the Engineered Soil, or Media, in the Biofilter can be entered using default values or, as User Defined values.

Enter the fraction for each soil type.  Then select “Apply All Values”.  Select “Continue”.

When the boxes next to “Apply” are checked, the program will use the calculated values in the Biofilter form.  The values will not appear until an Engineered Soil Depth is entered.

Run the model.

Results

Runoff Volume without controls:			3,181,000 ft3 per 5 yrs

Outfall Total with controls:			446,705 ft3 for 1 year (2,223,000 ft3 in 5 years)

Runoff Volume Percent Reduction:		30%

Particulate Solids Concentration (with controls):	18 mg/L

Particulate Solids Yield (with controls):		502 lbs/yr

Particulate Solids Percent Reduction:		81%

Rv (with controls):				0.30

Approx. Urban Stream Classification:		Poor

Total Phosphorus Concentration (with controls):	0.58

Total Phosphorus Yield (with controls):		80 lbs/5 yrs

Total Phosphorus Yield Percent Reduction:	69%

Annualized Value of all costs:			$10,005

The pollution reduction reported at the outfall is the overall pollution reduction for the entire site.

To see the pollution reduction from just the Biofilters, select the “Control Practices” tab.

The Biofilters are reducing the runoff volume by 30% and the TSS load by 81% from the source areas in the Commercial 1 land use. The underdrain short-circuits the infiltration potential (relatively low for this site) but provides substantial filtering of the stormwater before it is discahrges.

ET monitoring stations example (from Pitt, et al. 2009).

(use Station 5 (Oakmulgee, AL) for Birmingham and Tuscaloosa and station 6 (Bankhead, AL) for Huntsville)

| Map ID | State | Lat | Long | Elev | Station Name | Years of Data | ASCE Standardized Reference Evaporation Equation (ETo) (inday) |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Jan | Feb | Mar | Apr | May | Jun | Jul | Aug | Sep | Oct | Nov | Dec |
| 1 | AK | 64.84 | -147.62 | 454 | Fairbanks Alaska |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 | AK | 61.08 | -149.73 | 1480 | Rabbit Creek Alaska |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 | AK | 57.8 | -135.13 | 450 | Hoonah Alaska |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4 | AL | 33.44 | -86.081 | 600 | Talladega Alabama | 5 | 0.04 | 0.08 | 0.11 | 0.15 | 0.16 | 0.16 | 0.16 | 0.15 | 0.14 | 0.12 | 0.07 | 0.05 |
| 5 | AL | 32.96 | -87.171 | 363 | Oakmulgee Alabama | 7 | 0.05 | 0.06 | 0.10 | 0.13 | 0.15 | 0.16 | 0.15 | 0.14 | 0.14 | 0.10 | 0.07 | 0.05 |
| 6 | AL | 34.14 | -87.362 | 804 | Bankhead Alabama | 7 | 0.04 | 0.08 | 0.11 | 0.17 | 0.16 | 0.17 | 0.16 | 0.16 | 0.14 | 0.12 | 0.07 | 0.05 |
| 7 | AL | 32.45 | -85.641 | 283 | Tuskegee Alabama | 5 | 0.05 | 0.08 | 0.12 | 0.16 | 0.17 | 0.18 | 0.17 | 0.15 | 0.15 | 0.12 | 0.08 | 0.04 |
| 8 | AR | 34.76 | -90.722 | 253 | Marianna Arkansas | 3 | 0.04 | 0.06 | 0.11 | 0.15 | 0.16 | 0.20 | 0.18 | 0.17 | 0.14 | 0.10 | 0.07 | 0.04 |
| 9 | AR | 34.27 | -92.393 | 270 | Sheridan Arkansas | 6 | 0.04 | 0.07 | 0.12 | 0.08 | 0.19 | 0.20 | 0.21 | 0.19 | 0.17 | 0.11 | 0.08 | 0.04 |
| 10 | AR | 36.07 | -93.357 | 2365 | Compton Arkansas | 2 | 0.04 | 0.06 | 0.11 | 0.16 | 0.17 | 0.22 | 0.18 | 0.19 | 0.15 | 0.15 | 0.09 | 0.05 |
| 11 | AR | 35.87 | -94.297 | 1633 | Strickler Arkansas | 6 | 0.06 | 0.06 | 0.13 | 0.17 | 0.16 | 0.19 | 0.21 | 0.22 | 0.17 | 0.13 | 0.11 | 0.06 |
| 12 | AZ | 32.4 | -110.27 | 4175 | Muleshoe Ranch AZ | 13 | 0.07 | 0.13 | 0.17 | 0.24 | 0.28 | 0.30 | 0.24 | 0.22 | 0.23 | 0.20 | 0.12 | 0.08 |
| 13 | AZ | 35.15 | -111.68 | 7000 | Flagstaff Arizona | 10 | 0.04 | 0.07 | 0.11 | 0.16 | 0.20 | 0.24 | 0.22 | 0.18 | 0.17 | 0.13 | 0.07 | 0.04 |
| 14 | AZ | 32.32 | -110.81 | 3100 | Saguaro Arizona | 8 | 0.09 | 0.14 | 0.17 | 0.24 | 0.28 | 0.29 | 0.25 | 0.22 | 0.24 | 0.19 | 0.13 | 0.08 |
| 15 | AZ | 34.2 | -112.14 | 2960 | Sunset Point Arizona | 13 | 0.13 | 0.17 | 0.29 | 0.29 | 0.36 | 0.40 | 0.42 | 0.44 | 0.42 | 0.30 | 0.19 | 0.14 |
| 16 | CA | 37.79 | -122.14 | 1095 | Oakland South CA | 5 | 0.05 | 0.07 | 0.11 | 0.15 | 0.18 | 0.21 | 0.22 | 0.19 | 0.16 | 0.12 | 0.07 | 0.05 |
| 17 | CA | 34.13 | -118.41 | 1260 | Beverly Hills California | 6 | 0.07 | 0.08 | 0.10 | 0.14 | 0.14 | 0.15 | 0.12 | 0.12 | 0.10 | 0.08 | 0.08 | 0.07 |
| 18 | CA | 34.46 | -119.65 | 1500 | Montecito California | 6 | 0.08 | 0.08 | 0.11 | 0.15 | 0.17 | 0.19 | 0.18 | 0.18 | 0.15 | 0.12 | 0.11 | 0.07 |
| 19 | CA | 32.69 | -116.97 | 425 | San Miguel California | 5 | 0.07 | 0.09 | 0.11 | 0.15 | 0.16 | 0.17 | 0.18 | 0.18 | 0.14 | 0.12 | 0.08 | 0.06 |
| 20 | CA | 34.29 | -118.81 | 914 | Simi Valley California | 9 | 0.10 | 0.09 | 0.11 | 0.15 | 0.15 | 0.17 | 0.20 | 0.17 | 0.16 | 0.13 | 0.10 | 0.08 |
| 21 | CO | 39.48 | -105.21 | 8725 | Waterton North CO | 9 | 0.04 | 0.07 | 0.10 | 0.15 | 0.17 | 0.20 | 0.19 | 0.18 | 0.16 | 0.13 | 0.07 | 0.04 |
| 22 | CO | 40.57 | -105.23 | 6082 | Redstone Colorado | 11 | 0.04 | 0.04 | 0.06 | 0.07 | 0.13 | 0.15 | 0.16 | 0.13 | 0.13 | 0.05 | 0.05 | 0.03 |
| 23 | CO | 38.66 | -104.85 | 6700 | Ft. Carson Colorado | 10 | 0.05 | 0.08 | 0.11 | 0.16 | 0.18 | 0.22 | 0.21 | 0.18 | 0.17 | 0.13 | 0.08 | 0.05 |
| 24 | DE | 38.74 | -75.415 | 50 | Redden Delaware | 6 | 0.04 | 0.05 | 0.11 | 0.14 | 0.18 | 0.18 | 0.18 | 0.15 | 0.13 | 0.10 | 0.08 | 0.06 |
| 25 | FL | 25.63 | -80.58 | 5 | Chekika Florida | 8 | 0.11 | 0.14 | 0.17 | 0.21 | 0.20 | 0.18 | 0.16 | 0.16 | 0.16 | 0.16 | 0.13 | 0.11 |
| 26 | FL | 29.11 | -81.63 | 61 | Central Florida | 6 | 0.08 | 0.12 | 0.15 | 0.19 | 0.18 | 0.18 | 0.16 | 0.16 | 0.15 | 0.14 | 0.10 | 0.08 |
| 27 | FL | 30.01 | -84.424 | 50 | St. Marks (West) Florida | 7 | 0.10 | 0.11 | 0.13 | 0.14 | 0.14 | 0.15 | 0.15 | 0.13 | 0.13 | 0.12 | 0.10 | 0.10 |
| 28 | GA | 30.92 | -81.429 | 25 | Stafford-CUIS Georgia | 5 | 0.07 | 0.09 | 0.13 | 0.19 | 0.17 | 0.17 | 0.16 | 0.14 | 0.14 | 0.13 | 0.09 | 0.07 |
| 29 | GA | 32.1 | -81.083 | 10 | Savannah NWR SC | 6 | 0.06 | 0.09 | 0.13 | 0.17 | 0.19 | 0.18 | 0.17 | 0.14 | 0.14 | 0.12 | 0.08 | 0.06 |
| 30 | GA | 33.21 | -83.714 | 476 | Oconee #1 Georgia | 9 | 0.04 | 0.09 | 0.10 | 0.14 | 0.17 | 0.17 | 0.17 | 0.14 | 0.11 | 0.11 | 0.08 | 0.06 |
| 31 | GA | 32.01 | -82.9 | 250 | McRae Georgia | 6 | 0.06 | 0.09 | 0.12 | 0.17 | 0.18 | 0.18 | 0.16 | 0.15 | 0.15 | 0.13 | 0.08 | 0.06 |
| 32 | GA | 32.39 | -82.037 | 99 | Metter Georgia | 7 | 0.06 | 0.09 | 0.12 | 0.16 | 0.17 | 0.18 | 0.18 | 0.15 | 0.16 | 0.12 | 0.08 | 0.06 |
| 33 | GA | 33.9 | -83.366 | 675 | Watkinsville Georgia | 6 | 0.06 | 0.07 | 0.11 | 0.12 | 0.14 | 0.18 | 0.18 | 0.16 | 0.15 | 0.12 | 0.08 | 0.05 |
| 34 | GA | 32.01 | -84.33 | 526 | Plains/Sumter Georgia | 6 | 0.07 | 0.09 | 0.13 | 0.16 | 0.17 | 0.17 | 0.17 | 0.15 | 0.14 | 0.12 | 0.10 | 0.07 |
| 35 | GA | 31.71 | -82.388 | 109 | Baxley Georgia | 7 | 0.06 | 0.09 | 0.12 | 0.14 | 0.17 | 0.18 | 0.16 | 0.14 | 0.14 | 0.12 | 0.08 | 0.06 |

ASCE Standardized Reference ETo (in/day)

| Month | Station 5 (Oakmulgee, AL) | Station 6 (Bankhead, AL) |
| --- | --- | --- |
| January | 0.05 | 0.04 |
| February | 0.06 | 0.08 |
| March | 0.10 | 0.11 |
| April | 0.13 | 0.17 |
| May | 0.15 | 0.16 |
| June | 0.16 | 0.17 |
| July | 0.15 | 0.16 |
| August | 0.14 | 0.16 |
| September | 0.14 | 0.14 |
| October | 0.10 | 0.12 |
| November | 0.07 | 0.07 |
| December | 0.05 | 0.05 |