Cistern/Water Tank Example

For this example, we will start with the model file we created for No Controls and add water tanks/cisterns. Only the roof runoff will be routed to the water tanks and the water will be used to irrigate the large landscaped area.

Open the No Controls model file and Save the File with a new name.

Change the Site Description in the Current File Data to reflect the tanks.

Select the CI (cistern option) as the first control device for roofs 1/

The water use will be based on maximum applications to the large landscaped area (turf grass). This area is 2.14 acres and the monthly water use (source area gallons/day for each month) was calculated to be (almost all of the water use would occur from May through October at all three of these locations, requiring relatively large tanks to hold the winter runoff for this other period):

|  | Jan | Feb | Mar | Apr | May | Jun | Jul | Aug | Sep | Oct | Nov | Dec | Total Annual |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Average Birmingham monthly rain (in/mo) | 5.46 | 4.51 | 5.26 | 3.95 | 3.89 | 3.92 | 4.06 | 3.56 | 4.55 | 3.64 | 4.77 | 3.65 | 51.22 |
| Lawn moisture needs (in/mo) | 2 | 2 | 4 | 4 | 8 | 8 | 10 | 10 | 10 | 8 | 4 | 2 | 72 |
| Deficit irrigation need (in/mo) | 0 | 0 | 0 | 0.05 | 4.11 | 4.08 | 5.94 | 6.44 | 5.45 | 4.36 | 0 | 0 | 30.42 |
| Deficit irrigation needed (gallons/day) for 2.14 acres | 0 | 0 | 0 | 94 | 7,865 | 7,790 | 11,352 | 12,301 | 10,417 | 8,335 | 0 | 0 |  |

|  | Jan | Feb | Mar | Apr | May | Jun | Jul | Aug | Sep | Oct | Nov | Dec | Total Annual |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Average Tuscaloosa monthly rainfall | 4.25 | 3.29 | 4.60 | 4.87 | 4.00 | 3.00 | 4.52 | 3.22 | 3.37 | 4.04 | 3.07 | 5.75 | 47.98 |
| Lawn moisture needs (in/mo) | 2 | 2 | 4 | 4 | 8 | 8 | 10 | 10 | 10 | 8 | 4 | 2 | 72 |
| Deficit irrigation need (in/mo) | 0 | 0 | 0 | 0 | 4.00 | 5.00 | 5.48 | 6.78 | 6.63 | 3.96 | 0.93 | 0 | 32.78 |
| Deficit irrigation needed (gallons/day) for 2.14 acres | 0 | 0 | 0 | 0 | 7,646 | 9,547 | 10,479 | 12,959 | 12,668 | 7,574 | 1,778 | 0 |  |

|  | Jan | Feb | Mar | Apr | May | Jun | Jul | Aug | Sep | Oct | Nov | Dec | Total Annual |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Average Huntsville monthly rainfall | 5.26 | 4.77 | 6.65 | 4.68 | 5.03 | 4.25 | 4.55 | 3.43 | 4.08 | 3.43 | 4.73 | 5.49 | 56.34 |
| Lawn moisture needs (in/mo) | 2 | 2 | 4 | 4 | 8 | 8 | 10 | 10 | 10 | 8 | 4 | 2 | 72 |
| Deficit irrigation need (in/mo) | 0 | 0 | 0 | 0 | 2.97 | 3.75 | 5.45 | 6.57 | 5.92 | 4.57 | 0 | 0 | 29.24 |
| Deficit irrigation needed (gallons/day) for 2.14 acres | 0 | 0 | 0 | 0 | 5,681 | 7,166 | 10,418 | 12,559 | 11,324 | 8,736 | 0 | 0 |  |

With the CI form open, enter the data shown below. It can be re-opened by clicking on the CI label at the source area.

Note: when moving through the Cistern form, press the “Enter” key to move to the next cell, not the “Tab” key.

The water tank is described with the CI device, using the above water usage values:

Run the model.

Results

Years in model run:				4.98 (5 years)

Runoff Volume without controls:			3,181,000 ft3 for 5 years

Outfall Total with controls:			3,047,000 ft3 for 5 years (612,501 ft3 per year)

Runoff Volume Percent Reduction:		4.2%

Particulate Solids Concentration (with controls):	70 mg/L (an increase compared to no controls as better

quality roof runoff is being diverted from the stormwater

flows for the total area)

Particulate Solids Yield (with controls):		2,691 lbs/yr

Particulate Solids Percent Reduction:		0.4% (very little particulate solids in roof runoff)

Rv (with controls):				0.41

Approx. Urban Stream Classification:		Poor

Total Phosphorus Concentration (with controls):	1.34

Total Phosphorus Yield (with controls):		255 lbs/5 yrs

Total Phosphorus Yield Percent Reduction:	0.7%

Annualized Value of all costs:			not available

The pollution reduction reported at the outfall is the overall pollution reduction for the entire site.

To see the pollution reduction from just the roof runoff due to the water tanks, select the “Control Practices” tab.

The water tank and associated irrigation of the large landscaped area is reducing the roof runoff and particulate discharges by 24%. However, the outfall runoff reductions are much less as the roof runoff is only about 16% of the total site runoff (the paved parking area produces about 73% of the total site runoff in contrast).

The following table shows the effects of additional storage on roof runoff reductions that better utilize this water for on-site irrigation. The single tank only provides about 0.3 inches of storage over the roof area. About 1 ft of storage would be needed to provide about 75% roof runoff control, while about 2 ft of storage would result in about 100% removal of the roof runoff.

| Number of tanks (10ft D; 10 ft H) | Total storage volume (ft3) | Storage (ft3 storage/ft2 roof area) | Percent reduction of annual roof runoff | Percent reduction of total annual area runoff |
| --- | --- | --- | --- | --- |
| 1 | 800 | 0.027 | 24.1 | 4.2 |
| 3 | 2,400 | 0.081 | 36.3 | 6.3 |
| 5 | 4,000 | 0.14 | 41.7 | 7.3 |
| 10 | 8,000 | 0.27 | 47.8 | 8.3 |
| 25 | 20,000 | 0.68 | 62.2 | 10.9 |
| 50 | 40,000 | 1.35 | 78.2 | 13.6 |
| 75 | 60,000 | 2.03 | 100 | 16.0 |

Since the annual roof runoff is only a small portion of the total area annual runoff, complete removal of the roof runoff is therefore limited to a maximum of about 16%, even with the largest tanks. Therefore, it will also be necessary to reduce the flows from the paved parking areas with the use of biofilters, porous pavement (in overflow parking areas) or additional water tanks.